Read-me File For 'SyTest.EXE'.

Online Control Pty Ltd, 26 August, 1992

- Simple Sy/Link write/read test routine.

1. DISK CONTENTS:

SYTEST.EXE       DOS Executable

SYTEST.CFG       (ASCII) Configuration File

SYTEST.DOC       (this file)

2. PRELIMINARY:

Ensure that the card address set in 'SYTEST.CFG' corresponds to

your Sy/Link switch settings.

Note that the address, CF00 has worked in all A/T's tested.

The route has been initially set to 255 to communicate directly

with the Sy/Link mailbox.

If necessary, modify the value of 'PLCDROP' to communicate with

an external PLC. Note that the route for a PLC attached directly

to the Sy/Link 422 port is addressed as '1nn', where 'nn'

corresponds to the value of 'MYDROP'.

For example, if 'MYDROP' is set to '1', 'PLCDROP' would be set to

'101'.

2. SYNTAX:

From the DOS command prompt, type:

SYTEST [Enter]

Only individual register reads and writes are used. These are

much slower than the block reads & writes normally used for

PC/PLC communications but they provide a more thorough test of

the link.

--- oooOOOooo ---

Mario Galea

Senior Systems Engineer